Power Grid Simulator

Background

You're the systems engineer at a remote Arctic research station, 800km from the nearest town. The station runs on a single generator powering life support, heating, satellite communications, and lab equipment for 40 researchers. Outside, it's minus 35 degrees. If the generator overloads and the breaker trips, heating shuts down. At these temperatures, you have roughly 90 minutes before indoor pipes freeze and burst. Last winter, a faulty frequency reading caused an unnecessary breaker trip at 2am. By the time the on-call engineer woke up and reset the system, three labs had lost samples worth two years of data. Your job is to build a simulator that models this power system so the team can train, test, and rehearse exactly these scenarios before the next winter season.

Objective

Build a simulator for the station's single-bus power distribution system that models the physical process, implements the control logic, and displays the system state graphically.

System Components

- Generator G101: the station's sole diesel generator

- Circuit breaker CB101: connects or disconnects the station load from the bus

- Station load L101: everything the station needs to stay alive (heating, life support, labs, comms)

- Frequency sensor FS101: measures the bus frequency

Design Assumptions

- The system operates at a nominal frequency of 50.00 Hz. Frequency above 50.50 Hz is considered an over-frequency condition and below 49.50 Hz is considered an under-frequency condition.

- Under normal operation, the system should never enter an over-frequency or under-frequency condition.

- CB101 should never be CLOSED (load connected) when the frequency is below 49.50 Hz.

- The station always needs power. Therefore, CB101 should be CLOSED whenever feasible.

- Frequency variation rates are as follows:
a. G101 = ON, CB101 = CLOSED: frequency drops at 0.05 Hz/second (station load drawing power)
b. G101 = ON, CB101 = OPEN: frequency rises at 0.12 Hz/second (no load, generator spinning freely)
c. G101 = OFF, CB101 = CLOSED: frequency drops at 0.15 Hz/second (load draining stored energy)
d. G101 = OFF, CB101 = OPEN: no change in frequency

Functional Requirements

- Implement the simulator with control logic that maintains the bus frequency within safe operating bounds (between 49.50 Hz and 50.50 Hz). The simulator should run in discrete time steps (per second) over a configurable duration.

- Display the following as graphs over the simulation duration:
a. Bus frequency as reported by FS101, with the upper and lower limits clearly indicated
b. State of generator G101 (ON/OFF) over time
c. State of circuit breaker CB101 (OPEN/CLOSED) over time

AI-Assisted Development

The use of AI-assisted development is encouraged. You are free to use any GenAI model or tool of your choice. The use of open-source models running locally of your system (such as Qwen, GLM, DeepSeek) would be viewed favorably.

Please include an AI_USAGE.md file containing:

- The model or tool used and how it was accessed

- The prompts provided to the model

- The corresponding responses generated by the model

- Any modifications you made to the AI-generated output and why

Submissions that include AI_USAGE.md will be evaluated on how effectively the candidate used AI as a development tool.

Technical Requirements

- You may use Python or any programming language of your choice.

- Organize your project in a clear and maintainable manner.

- Document any assumptions made during development.

Deliverables

Submit a GitHub repository containing the complete project with the following:

- Source code

- README.md with project overview, installation instructions, and steps to run the simulator

- AI_USAGE.md documenting your use of GenAI (if applicable)

Share your GitHub repository link with us by replying to this email.

What We're Looking For

We're not expecting perfection. Nobody builds flawless systems on the first try, and that's not what this task is about. What we care about is how you think through a problem, how you use the tools available to you (including AI), and most importantly, how you learn from what doesn't work. The best engineers we've worked with aren't the ones who got everything right the first time. They're the ones who broke things, understood why, and came back with something better.

Show us your process. If something didn't work and you fixed it, tell us about it. If the AI gave you garbage and you had to rethink your approach, that's worth more to us than clean code that you can't explain. Build it, break it, fix it, and tell us the story.